DESIGN A 8-BIT ALU USING LOGISIM

A Project Report

Submitted by

CHINTADA PRADNISH - AP23110011198 GUNVANTH REDDY- AP23110010505 LOKESHWAR REDDY - AP23110010722 KRISHNA TRIPATHI - AP23110010408

MOURJO BASU- AP23110011139

Under the Supervision of

Mr. L. Srikanth Assistant Professor

Department of Computer Science and Engineering SRM University-AP

In partial fulfilment for the requirements of the

Computer Organization and Architecture(CSE305) Project

BACHELOR OF TECHNOLOGY IN

COMPUTER SCIENCE AND ENGINEERING

DEPARTMENT OF COMPUTER SCIENCE AND ENGINEERING SRM UNIVERSITY-AP

NEERUKONDA

MANAGALAGIRI - 522503 ANDHRA PRADESH,

INDIA December-2025

CERTIFICATE

This is to certify that the project work entitled “[Title of Your Project]” is a Bonafide record of project work carried out by the following students:

- Mr. Student-1 (Roll No.: AP23110011198)

- Mr. Student-2 (Roll No.: AP23110010505)

- Mr. Student-3 (Roll No.: AP23110010722)

- Mr. Student-4 (Roll No.: AP23110010408)

- Mr. Student-5 (Roll No.: AP23110011139)

from the Department of Computer Science and Engineering, SRM University-AP. The students conducted this project work under my supervision during the period August 2025 to December 2025.It is further certified that, to the best of my knowledge, this project has not previously formed the basis for the award of any degree or any similar title to this or any other candidate.

This is also to certify that the project work represents the teamwork of the candidates.

Station: Mangalagiri Mr. L. Srikanth Date: 14-10-2025 Assistant Professor Department of

Computer Science & Engineering

SRM

University-AP Andhra Pradesh.

TABLE OF CONTENTS

| S. No. | Title | Page No. |
| --- | --- | --- |
| 1 | Abstract | 1 |
| 2 | Introduction | 1 |
| 3 | Literature Survey | 2 |
| 4 | Problem Statement | 2 |
| 5 | Objective of the Project | 3 |
| 6 | System Requirements | 4 |
| 7 | Methodology / System Design | 5 |
| 8 | Implementation | 9 |
| 9 | Results & Discussion | — |
| 10 | Future Enhancements | — |
| 11 | Conclusion | — |
| 12 | References | — |
| 13 | Demo Video Link | — |
| 14 | GitHub Repository | — |
| 15 | Plagiarism Report | — |

- Abstract

This project focuses on developing an 8-bit Arithmetic Logic Unit (ALU) using Logisim, a graphical simulation tool widely used for experimenting with digital circuits. The ALU functions as the computational backbone of a processor, and in this design it is capable of executing essential arithmetic and logical operations such as addition, subtraction, AND, OR, and NOT. To achieve this, the system is constructed using core digital components including ripple-carry adders, basic logic gates, multiplexers, and organized wiring structures. These components are grouped into independent, modular sections so that each operation can be designed, tested, and validated separately before being integrated into the complete ALU.

The main aim of the project is to develop a solid understanding of how arithmetic and logical units operate inside actual computing systems and to enhance practical skills in combinational logic design. By building the system step-by-step, the project demonstrates how simple digital elements can be interconnected to form a fully functional subsystem. Once assembled, the ALU is tested under multiple input conditions to ensure reliability and correctness. This project not only showcases how an ALU works internally but also reinforces essential principles in digital electronics and processor architecture.

- Introduction

The Arithmetic Logic Unit (ALU) is a key hardware component found in all modern processors. It performs the crucial arithmetic and logical computations required for executing instructions, making decisions, and managing program flow. Whether it is adding two binary values or evaluating logical expressions, the ALU is the primary unit responsible for these fundamental operations. Its effectiveness directly influences the performance and capability of the entire computing system.

In this project, an 8-bit ALU is designed and implemented using Logisim, a beginner-friendly digital logic simulator that allows circuit designers to visualize and test digital systems. The purpose of the project is to gain hands-on experience by constructing an ALU using basic components such as full adders, multiplexers, logic gates, and bus structures. Building the ALU from scratch illustrates how complex operations originate from simpler ones, and Logisim’s visual environment helps in observing signal flow and verifying circuit behavior in real time.

- Literature Survey

Foundational works in digital logic and computer architecture—such as Patterson and Hennessy’s Computer Organization and Design and Mano’s Digital Logic and Computer Design—provide detailed explanations of how arithmetic and logic functions are implemented using hardware. These books describe the theory behind binary addition and subtraction, Boolean operations, and the internal circuitry used to realize these operations, such as full adders, combinational logic gates, multiplexers, and structured data paths. They also introduce standard ALU architectures, which form the conceptual basis for modern processor design.

Apart from textbooks, several academic resources, online courses, and practical tutorials demonstrate ALU design using tools like Logisim. These references commonly adopt a modular design approach—starting with simple elements such as ripple-carry adders for the arithmetic unit and arrays of logic gates for bitwise operations. Multiplexers are frequently used to choose the required output based on control signals. This stepwise method is widely used in educational environments because it is easy to understand and aligns well with real digital hardware structures.

The design of this ALU was shaped by the techniques and methodologies discussed in these resources. By following well-established approaches, the project ensures a reliable, logically structured, and educationally meaningful ALU design.

- Problem Statement

To design and implement an 8-bit Arithmetic Logic Unit (ALU) using Logisim that can perform key arithmetic and logical operations. The ALU must accept two 8-bit inputs and use a 3-bit control signal to determine the selected operation. The system should be modular, easy to interpret, and fully testable within the simulation environment.

- Objectives of the Project

● Design a complete 8-bit ALU in Logisim.
Build a functional digital circuit capable of performing both arithmetic and logical operations.

● Implement essential arithmetic functions: addition and subtraction.
Use full adders and two’s complement logic to ensure correct processing of 8-bit values.

● Implement key logic functions: AND, OR, and NOT.
Use arrays of identical logic gates to perform bitwise operations across all 8 bits.

● Use a multiplexer for operation selection.
The multiplexer will select the appropriate result based on the 3-bit control code.

● Strengthen understanding of combinational logic and modular design.
Develop the ALU step-by-step using small components to build familiarity with hardware organization.

● Test and validate the complete ALU.
Ensure every operation works correctly across various input combinations and produces accurate results.

- System Requirements

Hardware Requirements

- A computer or laptop capable of running Logisim.

Software Requirements

- Logisim or Logisim-Evolution

- Windows or macOS operating system

Functional Requirements

- Two 8-bit input buses: A and B

- One 3-bit control input

- One 8-bit output

- Components needed:

  - 8-bit adder

  - Basic logic gates (AND, OR, NOT, XOR)

  - Multiplexers

  - Splitters and wiring tools

- Methodology / System Design

Step 1: Creating Input Buses

Two 8-bit inputs (A and B) and a 3-bit control input are added to serve as the primary data and selection lines.

Step 2: Designing the Arithmetic Section

An 8-bit ripple-carry adder is created using eight 1-bit full adders.

- Addition: A + B

- Subtraction: A + (two’s complement of B)

Step 3: Designing the Logic Section

Bitwise operations are implemented using sets of identical gates:

- 8 AND gates

- 8 OR gates

- 8 NOT gates

Each produces an 8-bit output.

Step 4: Multiplexer-Based Output Selection

A multiplexer determines which output is sent to the final 8-bit display:

- 000 → Addition

- 001 → Subtraction

- 010 → AND

- 011 → OR

- 100 → NOT

Step 5: ALU Integration and Testing

All modules are connected using splitters and structured wiring. Various test inputs are applied to verify correct output behavior.

Step 6: Verification

Simulation results are compared with manually calculated values to ensure complete correctness.

Contribution

This project successfully demonstrates the design, simulation, and validation of a fully functional 8-bit ALU, which represents one of the most crucial components of a CPU. It achieves the following:

- Combines arithmetic (addition, subtraction) and logic (AND, OR, XOR, NOT) operations into a unified architecture.

- Shows how multiplexers can effectively control data flow within a processor.

- Provides a scalable model that can be expanded for more advanced processor designs.

Implementation

The ALU was built in Logisim-Evolution by dividing the design into separate modules. Input buses and a 3-bit selector were created first. The arithmetic module was constructed using an 8-bit ripple-carry adder. Subtraction was implemented using two’s complement logic.

The logic section was developed using arrays of eight identical gates for each operation. A multiplexer was added to choose which result would appear on the final output bus. After wiring and organization using splitters, the complete circuit was tested with numerous input values.

Results & Discussion

Simulation results confirmed that the ALU performed all operations correctly.

- The ripple-carry adder handled carries properly during addition.

- Two’s complement subtraction produced accurate results for all test cases.

- Logical operations matched expected truth-table outputs.

- The multiplexer switched cleanly between different functional blocks.

The modular approach allowed easy debugging and ensured reliable performance during integration. The results demonstrate a stable and well-structured ALU design.

Future Enhancements

Potential improvements include:

● Extending the design to 16-bit or 32-bit ALUs
● Adding more operations (XOR, NAND, NOR, XNOR, INC, DEC, CMP)
● Implementing shift and rotate operations
● Adding status flags (Carry, Zero, Negative, Overflow)
● Integrating the ALU into a basic CPU architecture
● Improving readability with labeled buses and cleaner layout

Conclusion

This project provided meaningful hands-on experience in building a core component of a processor—the Arithmetic Logic Unit. Constructing the ALU from the ground up demonstrated how simple elements like full adders, logic gates, and multiplexers combine to create a powerful computational unit. Each operation behaved accurately during testing, showing the effectiveness of the modular building approach.

Beyond completing the circuit, this project helped deepen understanding of how processors perform calculations and make decisions. It serves as a strong foundation for learning more advanced topics in computer architecture, digital system design, and embedded systems. The 8-bit ALU created here is not only functional but also scalable and educationally valuable.

# References

- M. Morris Mano, Digital Logic and Computer Design, Pearson Education.

- David A. Patterson & John L. Hennessy, Computer Organization and Design, Morgan Kaufmann.

- Logisim-Evolution Project Documentation – GitHub.

- Neso Academy – Digital Electronics Tutorials.

- Charles H. Roth, Fundamentals of Logic Design, Cengage.